Begränsad delning

VOLTARC ENERGY SYSTEMS

"Power Without Compromise"

System Manual: CellSentinel-12 Battery Management System

Document ID: VES-CS12-MAN-R2B

Internal Project Code: "Project Thunderstone"

Status: Production / ISO 26262 ASIL-C Certified

# 1. Introduction: The Voltarc Philosophy

The CellSentinel-12 monitors up to 216 series-connected lithium-ion cells, performs real-time State-of-Charge estimation, and enforces thermal and voltage protection with sub-millisecond contactor trip response. It doesn’t just watch the battery; it keeps it alive.

Global Support Liaison:

- Contact: Dr. Leena Voss

- Email: l.voss@voltarc-energy.com

- Note: "A battery pack without a BMS is just a very expensive, very dangerous space heater."

# 2. The Physical Architecture

The CellSentinel-12 is distributed across a central Supervisor Module (SM-1) and up to 18 daisy-chained Cell Monitoring Satellites (CMS-12), each watching 12 cells.

## 2.1 Supervisor Module (SM-1)

The brain of the pack. Handles SoC/SoH estimation and communicates with the vehicle ECU.

- Processor: Infineon AURIX TC397 tri-core, 300 MHz, lockstep.

- Contactor Control: Drives positive, negative, and pre-charge contactors. Pre-charge resistor: 47 Ω, 50 W ceramic. Time constant: τ = R·C = 47 × 0.0033 = 155 ms.

- Current Sensing: Hall-effect sensor, ±1,000 A range, ±0.5% accuracy.

- Isolation Monitoring: 500 Hz AC injection. Trip threshold: < 500 Ω/V.

## 2.2 Cell Monitoring Satellite (CMS-12)

Each satellite sits physically next to its 12-cell group inside the module.

- Cell Voltage ADC: 16-bit, ±0.8 mV accuracy. Conversion time: 290 µs for all 12 channels simultaneously. At 4.20 V the cell is full; at 4.25 V you’re measurably shortening its life.

- Temperature: 3 × NTC thermistors per satellite (10 kΩ at 25°C, β = 3,435 K).

- Passive Balancing: Bleed resistors, 33 Ω, ~120 mA at 4.0 V per cell.

- Communication: Isolated SPI at 1 MHz, daisy-chain. Full-pack snapshot latency: < 5 ms.

## 2.3 Thermal Management Interface (TMI-1)

- Coolant Valves: 2 × PWM outputs controlling glycol flow. Target: 8–12 L/min.

- Heater: 3 kW PTC, activates below 5°C. Draws from HV pack directly.

- Physics Note: Internal thermal model runs at 10 Hz with 3 nodes per cell (core, surface, tab). RMSE: ~1.2°C. Do NOT run NMC profiles on an LFP pack. It’ll look fine until it very much isn’t.

# 3. Technical Specifications Table

| Sub-System | Attribute | Value | Tolerance |
| --- | --- | --- | --- |
| Supervisor | Max Series Cells | 216 (18 × 12) |  |
| Supervisor | Pack Voltage Range | 180 – 870 V DC | 400V / 800V arch. |
| Supervisor | Contactor Trip | < 3 ms | Pyro-fuse: < 0.5 ms |
| CMS-12 | Voltage Accuracy | ±0.8 mV | At 25°C |
| CMS-12 | Balancing Current | ~120 mA | At 4.0 V |
| Current Sensor | Range | ±1,000 A | DC–50 kHz |
| Thermal | Heater Power | 3 kW | PTC element |
| Whole System | Operating Temp | −40 to +85°C | Electronics |

# 4. Operational State Logic

The CellSentinel-12 runs PackGuard™ firmware, a deterministic state machine with a 1 ms base tick.

- SLEEP: Power draw < 150 µA. Only CAN wake-up transceiver active. Wakes on CAN frame 0x100 or charger voltage detection.

- PRE-CHARGE: Negative contactor closes, then pre-charge. Bus must reach ≥ 95% of pack voltage within 2 seconds or all contactors open.

- OPERATIONAL: Full monitoring. Balancing enabled if ΔV > 15 mV and pack at rest. Thermal commands at 10 Hz.

- CHARGE: BMS provides real-time current limit to the charger based on worst-case cell. CC→CV transition at V_max. Termination: I_tail < C/20.

- FAULT: Cell overvoltage, undervoltage, overtemp, or isolation loss. Contactors open immediately. Latched — requires diagnostic tool reset.

- THERMAL RUNAWAY: Any thermistor > 90°C with dT/dt > 1°C/s fires the pyro-fuse (irreversible). Pack is a total loss. That is the correct outcome.

# 5. Maintenance & "Unstructured" Field Notes

- The 3 AM Phantom Drain: Fleet operators reported 2–3% SoC loss overnight. Root cause: a faulty door sensor was sending CAN wake frames. The battery was fine; the car was insomniac.

- Balancing on LFP is Painful: LFP cells have a flat voltage plateau (~3.25 V ± 10 mV) between 20–80% SoC. Passive balancing in this range is like levelling a lake with a teaspoon. Schedule monthly top-balance charges.

- Coolant Leak on J5: Early units (S/N below CS12-0042000) had undersized O-rings. Replace with VES-SEAL-0087 (Viton, 7.65 mm ID). Fixed from Sep 2025.

- Warning: Pack voltage is present on bus bars at all times, even with contactors open. Service disconnect must be physically removed. Min PPE: Class 0 insulating gloves, rated 1,000 V AC. If you’re reading this after the fact, consult a medical professional.

# 6. Component Connectivity Table

| Connector | Type | Signal / Voltage | Destination |
| --- | --- | --- | --- |
| J1 | Ampseal 35-pin | Iso-SPI Daisy-Chain | Cell satellites |
| J2 | HV Interlock | 24 V DC Sense | Pack cover / Service plug |
| J3 | Deutsch DT06-12S | CAN-FD (500k/2M) | Vehicle ECU |
| J4 | TE MCP 2.8 mm | 12 V Contactor Drive | +, –, Pre-charge |
| J5 | M8 4-pin | Coolant Valve PWM | Thermal valves |
| J6 | Anderson SB50 | HV Feed | PTC heater |
| J7 | Micro-Fit 3.0 | CAN (Charger) | EVSE / OBC |
| J8 | Pyro-fuse Trigger | Squib (redundant) | Pyro cartridge |

# 7. Troubleshooting "Quick-Start"

Issue: Dashboard shows "Replace Hybrid Battery." Solution: SoH dropped below threshold (default 70%). Check trend log via UDS DID 0xF190. If sudden knee in curve, suspect a single weak satellite — check per-module SoH via DID 0xF191.

Issue: Pre-charge timeout (DTC P0AA6). Solution: Either the pre-charge resistor is open (measure 47 Ω), the bus cap is shorted, or the positive contactor is welded shut. Start with a multimeter. Contactors do not weld themselves for fun.