Begränsad delning

LUMEN GRID SYSTEMS

"Every Light, Every Cycle, Every Time"

Technical Manual & Integration Guide: TL-Nexus 400 Adaptive Signal Controller

Document ID: LGS-TLN400-MAN-R3

Internal Project Code: "Project Chameleon"

Status: Released / Field-Proven (Mostly)

# 1. Executive Summary

The TL-Nexus 400 is not merely a traffic light; it is a real-time intersection arbitrator. Designed for urban junctions with up to 8 approaches, the system uses inductive loops, LiDAR, and thermal imaging to dynamically allocate green-phase durations while enforcing pedestrian safety corridors.

Note from the Chief Traffic Architect: "A traffic light that cannot adapt is just a very expensive Christmas decoration." — Dr. Henrik Solvang (h.solvang@lumengrid.io)

# 2. Component Breakdown & Physical Specs

The system is housed in a sealed IP67 aluminium enclosure. It consists of four primary subsystems: The Eye, The Brain, The Voice, and The Backbone.

## 2.1 The Eye (Sensor Array – Unit SA-4)

The sensor array gives a 360° awareness envelope around the intersection.

- Inductive Loops: 4-channel multiplexed, 20–80 kHz excitation. Detects ferromagnetic mass changes ≥ 0.02% of baseline inductance.

- LiDAR Module: Solid-state flash, 905 nm, Class 1 eye-safe. Range: 0.3–120 m. 20 Hz refresh.

- Thermal Camera: 640 × 512 VOx microbolometer. For pedestrian detection in fog, rain, and night.

- Acoustic Sensor: MEMS microphone array for emergency siren detection. Classification latency: < 120 ms.

## 2.2 The Brain (Decision Core – Unit DC-2)

The central logic runs the proprietary GreenWave™ firmware.

- Processor: ARM Cortex-R52 dual-core, 400 MHz, lockstep for ASIL-D.

- RAM: 512 KB tightly-coupled SRAM. No external DRAM — to eliminate bit-flip risk from cosmic rays. (Yes, really.)

- Cycle Time: Full intersection solve in ≤ 8 ms for 4 approaches.

- Watchdog: Hardware, 250 ms timeout. If the solver hangs, it falls back to a fixed-time plan.

## 2.3 The Voice (Signal Head Driver – Unit SH-8)

Controls up to 32 independent LED signal channels.

- Output: Constant-current, 350 mA per channel at 48 V DC.

- LED Wavelengths: Red: 625 nm, Amber: 590 nm, Green: 505 nm.

- Conflict Monitor: Hardwired analog relay board. Checks no two opposing greens are active. If conflict detected: all-red within 80 ms. Cannot be overridden by software. That is the point.

## 2.4 The Backbone (Comms – Unit CM-3)

- Primary Link: Ethernet 100BASE-TX via fibre, NTCIP 1202 protocol.

- Fallback: 4G LTE Cat-4 modem. Sufficient for telemetry, not video.

- Diagnostic Port: RS-232, 115200 baud. Default credentials on a sticker inside the cabinet. (Please change them.)

# 3. Technical Specifications Table

| Sub-System | Attribute | Value | Tolerance |
| --- | --- | --- | --- |
| Sensor Array | LiDAR Frame Rate | 20 Hz | ±0.5 Hz |
| Sensor Array | Loop Excitation | 20–80 kHz | Auto-tune |
| Decision Core | Solver Cycle | ≤ 8 ms | Worst-case 12 ms |
| Decision Core | Power Draw | 1.8 W typical | Peak |
| Signal Driver | LED Current | 350 mA/ch | ±5% |
| Signal Driver | Conflict Detect | 80 ms to all-red | Analog |
| Comms | Ethernet | 100 Mbps | Full duplex |
| Enclosure | Operating Temp | −40°C to +65°C | IP67 |
| Enclosure | Weight | 38.5 kg | ±0.5 kg |

# 4. Operational Logic & State Machine

The GreenWave™ state engine enforces MUTCD-compliant minimum green, yellow clearance, and all-red intervals. The system cannot skip a yellow phase.

- BOOT / SELF-TEST: ~4.5 seconds. All LED channels tested. Conflict board verified. Failure = immediate FLASH.

- FIXED-TIME (Fallback): Pre-loaded timing plan runs with constant cycle length (default 90 s). Used if sensors are offline.

- ADAPTIVE (Primary): Reads sensors every 100 ms, recalculates phase splits each cycle. Green extension: up to +15 s based on queue length. Max cycle: 180 s.

- PREEMPTION: Triggered by siren detection (confidence ≥ 92%). Active phase truncated; emergency vehicle gets unconditional green.

- FLASH: All red signals flash at 1 Hz. Entered on conflict, cabinet door open, or manual command. Will not leave FLASH without manual reset. If it’s flashing, something went wrong, and a human should look at it.

# 5. Maintenance & "Unstructured" Field Notes

- The "Ghost Cyclist": In cold weather, the thermal camera hallucinates a cyclist at the NW corner. It’s a steam vent from the subway grate. Add coordinates to the exclusion mask in TLN400.cfg, line 441.

- Loop Sealant: If a loop stops detecting, it’s water infiltration. Re-seal with bituminous sealant, NOT silicone caulk. Silicone expands in summer. We learned this in Tucson.

- Emergency Vehicle False Positives: The classifier was trained on North American and European sirens. If a bus honks and gets a green light, retrain using LGS-SIREN-TOOL. Do NOT just disable preemption.

- Cabinet Spiders: In rural installations, orb-weaver spiders love the warmth. Their webs short the conflict monitor contacts. Inspect quarterly. Engineering change request for factory mesh screens is pending.

- Warning: The 240 V mains contactor is behind the lower panel. Three technicians were shocked last fiscal year. If the label was insufficient motivation, let this sentence be additional encouragement.

# 6. Wiring & Connectivity Interface

| Terminal | Type | Signal / Voltage | Destination |
| --- | --- | --- | --- |
| TB-01 | RJ45 (Shielded) | 100BASE-TX Ethernet | ATCC via fibre |
| TB-02 | SMA Female | 4G LTE Antenna | Roof-mount omni |
| TB-03–TB-06 | 4-pin Molex | Inductive Loop Pairs | Roadway saw-cuts |
| TB-07 | M12 8-pin | LiDAR + Thermal | Mast-mounted head |
| TB-08 | 5-pin DIN | Conflict Monitor I/O | Analog relay board |
| TB-09 | DB-9 Male | RS-232 Diagnostic | Laptop |
| TB-10 | 3φ + N + PE | 240 V AC Mains | Utility supply |

# 7. Troubleshooting "Quick-Start"

Issue: All signals stuck on red, no flash. Solution: Conflict monitor forced all-red. Check SH-8 driver board for a stuck relay on Channel 14 (known issue, PCB rev. C3). Replacement relay: Omron G5V-1-DC48.

Issue: LiDAR reports 0 points, sensor LED solid green. Solution: The lens is covered in bird droppings. Clean with isopropyl alcohol. Consider the optional LGS-BIRDSPIKE kit (Part No. LGS-ACC-0091).

Issue: Controller reboots every 37 minutes exactly. Solution: A rogue SNMP poll from the ATCC sends a malformed OID that crashes the comms stack. Update CM-3 firmware to v2.4.1. Or ask the ATCC to fix their script. Good luck with that.